# STM32F446RE Peripheral Driver Analysis Report

Project: Bare-Metal Peripheral Driver Development for STM32F446RE
Drivers: GPIO, USART, SPI, I2C

# Introduction

This report explains the implementation and usage of the STM32F446RE bare-metal peripheral drivers. The project includes GPIO, USART, SPI and I2C drivers developed using direct register programming. Each driver encapsulates low-level hardware details and exposes simple APIs for application development.

# GPIO Driver

## 1. Introduction

GPIO is used to configure digital input/output pins for LEDs, switches and alternate peripheral functions.

## 2. Driver Structure

The driver consists of a header (.h) file containing macros, structures and API declarations, and a source (.c) file implementing the peripheral functionality.

## 3. Initialization

GPIO_Init() enables the GPIO peripheral clock and configures MODER, OTYPER, OSPEEDR, PUPDR and AFR registers based on the selected configuration.

## 4. APIs Offered

GPIO_Init()

GPIO_DeInit()

GPIO_ReadFromInputPin()

GPIO_ReadFromInputPort()

GPIO_WriteToOutputPin()

GPIO_WriteToOutputPort()

GPIO_ToggleOutputPin()

GPIO_IRQConfig()

GPIO_IRQHandling()

## 5. Hardware Abstraction

The application fills a GPIO_Handle_t structure and calls GPIO_Init(). The driver performs all required register configuration internally.

## 6. Example Application

LED blinking

## 7. Summary

The GPIO Driver provides reusable APIs that simplify application development and reduce direct register manipulation.

# USART Driver

## 1. Introduction

USART provides asynchronous serial communication for debugging and data transfer.

## 2. Driver Structure

The driver consists of a header (.h) file containing macros, structures and API declarations, and a source (.c) file implementing the peripheral functionality.

## 3. Initialization

USART_Init() configures baud rate, word length, parity, stop bits, mode and hardware flow control through USART registers.

## 4. APIs Offered

USART_Init()

USART_SendData()

USART_ReceiveData()

USART_SendString()

USART_PeripheralControl()

## 5. Hardware Abstraction

The driver hides BRR, CR1, CR2 and CR3 register programming. Applications only provide configuration values.

## 6. Example Application

Sending 'Button Pressed' messages and printing sensor data to Tera Term.

## 7. Summary

The USART Driver provides reusable APIs that simplify application development and reduce direct register manipulation.

# SPI Driver

## 1. Introduction

SPI enables high-speed synchronous communication with external peripherals.

## 2. Driver Structure

The driver consists of a header (.h) file containing macros, structures and API declarations, and a source (.c) file implementing the peripheral functionality.

## 3. Initialization

SPI_Init() configures master/slave mode, baud rate, CPOL, CPHA, data frame format and software slave management.

## 4. APIs Offered

SPI_Init()

SPI_SendData()

SPI_ReceiveData()

SPI_PeripheralControl()

## 5. Hardware Abstraction

Applications exchange data through API calls while the driver handles status flags and register access.

## 6. Example Application

Sending a test pattern to an SPI device.

## 7. Summary

The SPI Driver provides reusable APIs that simplify application development and reduce direct register manipulation.

# I2C Driver

## 1. Introduction

I2C communicates with external sensors, EEPROMs and RTC devices over SDA and SCL.

## 2. Driver Structure

The driver consists of a header (.h) file containing macros, structures and API declarations, and a source (.c) file implementing the peripheral functionality.

## 3. Initialization

I2C_Init() configures clock speed, ACK control, own address, CCR and TRISE registers for standard mode operation.

## 4. APIs Offered

I2C_Init()

I2C_MasterSendData()

I2C_MasterReceiveData()

I2C_PeripheralControl()

## Implementation of I2C_Master_Receive_Data

void I2C_MasterReceiveData(I2C_Handle_t *pI2CHandle,

uint8_t *pRxBuffer,

uint32_t Len,

uint8_t SlaveAddr,

uint8_t Sr)

{

I2C_GenerateStartCondition(pI2CHandle->pI2Cx);

I2C_ExecuteAddressPhaseRead(pI2CHandle->pI2Cx, SlaveAddr);

if(Len == 1)

{

pI2CHandle->pI2Cx->CR1 &= ~(1 << 10); // no ACK

I2C_GenerateStopCondition(pI2CHandle->pI2Cx);

while(!(pI2CHandle->pI2Cx->SR1 & (1 << 6)));

*pRxBuffer = pI2CHandle->pI2Cx->DR;

}

if(Len > 1)

{

for(uint32_t i = Len; i > 0; i--)

{

if(i == 2)

{

pI2CHandle->pI2Cx->CR1 &= ~(1 << 10);

I2C_GenerateStopCondition(pI2CHandle->pI2Cx);

}

while(!(pI2CHandle->pI2Cx->SR1 & (1 << 6)));

*pRxBuffer = pI2CHandle->pI2Cx->DR;

pRxBuffer++;

}

}

if(pI2CHandle->I2C_Config.I2C_ACKControl == I2C_ACK_ENABLE)

{

pI2CHandle->pI2Cx->CR1 |= (1 << 10);

}

}

Explanation

The I2C_MasterReceiveData() function is responsible for receiving data from an I²C slave device in master mode. It generates a START condition, sends the slave address with the read bit set, receives one or more bytes while monitoring the RXNE flag, manages ACK/NACK generation, and generates a STOP condition after the reception is complete. This function provides a simple API for reading data without requiring the application to manipulate I²C registers directly.

## 5. Hardware Abstraction

Applications specify the slave address and data buffer while the driver generates START/STOP conditions and transfers data.

## 6. Example Application

Reading temperature from an LM75 sensor example and transmitting it over USART.

## 7. Summary

The I2C Driver provides reusable APIs that simplify application development and reduce direct register manipulation.

# Conclusion

The developed GPIO, USART, SPI and I2C drivers provide a reusable hardware abstraction layer for the STM32F446RE. They simplify embedded application development by encapsulating register-level programming into modular APIs.